Question 1

The incompressible flow of a Newtonian fluid between two flat parallel plates, as shown in figure below, far from the entrance region, is governed by the following one-dimensional steady-state momentum equation:

ddyμdudy-dpdx=0,  0<y<H

where x and y denote, respectively, the horizontal and vertical directions, u is the flow speed along the horizontal direction and is a function of y only, μ is the (constant) dynamic viscosity of the fluid, H is the distance between the plates, and dpdx is the pressure gradient along the flow direction, which can be assumed as a constant source term. Therefore, the only independent variable in the equation above is y, and solution of the equation should yield the velocity profile u(y) between the top (y=H) and bottom (y=0) walls.

(a)	Derive the discretised form of the equation above using the finite-volume method, for a generic control volume located within the domain (i.e. not a boundary control volume). Treat the pressure gradient as a constant source term.

[10 marks]

(b)	Consider a channel of height H=0.02 m, meshed with n=5 control volumes (2 centred on the top and bottom boundaries and 3 internal ; centroids are equidistant), constant μ=0.001 kg/(m s), constant pressure gradient dpdx=-1000 Pa/m, and no-slip boundary conditions u(y=0)=0 and u(y=H)=0. Write down the matrix of the coefficients and the vector of the known terms for the entire mesh.

[6 marks]

(c)	Solve the linear system using the Gauss-Seidel iterative method, choosing appropriate initial guess values, and halting the procedure after 2 iterations. Provide a maximum 4 decimal places when writing down values.	[4 marks]

Question 2

Consider a one-dimensional, steady-state heat conduction problem where heat diffuses across a solid wall between the boundaries x=0 and x=L, see sketch in figure below. No heat generation is present. The problem is governed by the one-dimensional steady heat conduction equation, with no source terms, written as:

ddxλdTdx=0,  0<x<L.

At the boundary x=0, the temperature is fixed to a constant value Ta:

Tx=0=Ta.

At the boundary x=L, a constant heat flux is provided to the system, and thus via the Fourier heat equation:

dTdxx=L=q,

where, based on the figure below, q>0.

- Derive the discretised form of the heat conduction equation above using the finite-volume method as seen in the Numerical Methods part of the module, for a generic control volume located within the domain (i.e. not a boundary control volume). For simplicity, you can assume that the distance between the centroids of the control volumes is constant and that the thermal conductivity λ is constant. [10]

- Consider a computational grid where the centres of the two boundary control volumes coincide with the boundaries (x=0 and x=L), and derive the discretisation equations for these two boundary control volumes, according to the boundary conditions specified above. [7]

- Consider a domain of length L=0.1 m, meshed with n=5 control volumes (2 centred on the left and right boundaries and 3 internal; centroids are equidistant), constant λ=250 W∙m-1∙K-1, temperature at the left boundary Ta=290 K, heat flux at the right boundary q=10 kW∙m-2. Write down the matrix of the coefficients and the vector of the known terms (showing all the numerical values) for the entire mesh. [3]

Question 3

A fluid falls steadily through the gap formed between two vertical, parallel, stationary, infinitely extended walls, as sketched in Figure Q1. The steady-state momentum equation along the y-coordinate, for an incompressible and fully-developed flow of a Newtonian fluid (assuming the absence of a pressure gradient driving the flow), can be written as follows:

ddxμdvdx+ρg=0,  0<x<W                                                                                                            (1)

where x and y denote, respectively, the horizontal and vertical directions, v is the flow speed along the vertical direction and is a function of x only, μ is the (constant) dynamic viscosity of the fluid, and W is the distance between the plates.  is the density of the fluid and g the gravitational acceleration; their product can be assumed as a constant source term. Note that the only independent variable in Eq. (1) is x, and the solution of the equation should yield the velocity profile v(x) between the left (x=0) and right (x=W) walls.

(a) Derive the discretised form of Eq. (1) using the finite-volume method, for a generic control volume located within the domain (i.e. not a boundary control volume). 	[10]

(b) Consider a channel of width W=0.01 m meshed with n=5 control volumes (2 centred on the left and right boundaries and 3 internal with equidistant centroids). Assume μ=0.001 kg∙m-1∙s-1, r=1000 kg∙m-3 and gravitational acceleration g=-9.81 m∙s-2. Assume no-slip boundary conditions (i.e. v(x=0)=0 and v(x=W)=0). Write down the matrix of the coefficients and the vector of the known terms for the entire mesh. 		 									[5]

Question 4

Consider the following function for the one-dimensional temperature profile across a wall, in direction x:

Tx=x3+x2+2

Consider that the domain is discretised in control volumes, with xe=1, see figure below. The first-order derivative of T(x) in xe can be approximated with the following discretisation scheme:

dTdxxe=TxE-TxPδxe

Using δxe=0.2 and 0.4, demonstrate that the discretisation scheme above is second-order. 										[5 marks]

Question 5

If the matrix of the coefficients of a linear system deriving from the finite-volume discretisation of the steady heat conduction equation is penta-diagonal, explain whether the geometry of the problem is one- or two-dimensional, and whether the mesh is structured or sparse.							[5 marks]

Question 6

If the truncation error of a discretisation scheme is Oδx 2T ''', where T(x) is the function that the discresisation scheme approximates along direction x and δx is the distance between the nodes centres, explain what such an error means. [5 marks]

Question 7

The analysis of the convergence of the solution error in the solution of a one-dimensional heat-conduction problem upon refinement of the mesh returned the following values of the errors (E) for three different mesh resolutions (dx):

dx1=0.1, E1=5

dx2=0.2, E2=40

dx3=0.4, E3=320

What is the convergence order of the numerical algorithm?