Question 1

The one-dimensional unsteady heat conduction equation, neglecting the source term, writes as follows:

∂ρcpT∂t=∂∂xλ∂T∂x

where T is the temperature, t the time, ρ the density, cp the specific heat capacity at constant pressure, x the spatial 	coordinate and λ the thermal conductivity.

(a) Comment on the advantages of using a time-implicit scheme to discretise the time-derivative in the equation above. [5 marks]

(b) Derive the discretised form of the equation above using the finite-volume method, for a generic control volume located within the domain (i.e. not a boundary control volume), using a 1st-order time-implicit scheme. [10 marks]

(c) Derive the stability condition for this algorithm. [5 marks]

Question 2

The time-explicit temporal scheme is employed to solve a 1D unsteady heat conduction problem. The parameters of the problem are: domain length L=0.15 m, thermal conductivity λ=100 W/(m K), density ρ=2000kgm3, specific heat cp=1000 J/(kg∙K), Dirichlet boundary conditions Ta=Tb=300 K, initial condition T0=350 K, zero source term, and n=5 equidistant nodes. What is the largest time-step that ensures stable calculations? [5 marks]

Question 3

The one-dimensional unsteady heat conduction equation, with a non-zero source term, writes as follows:

∂ρcpT∂t=∂∂xλ∂T∂x+S(x,t,T)

where T is the temperature, t the time, ρ the density, cp the specific heat capacity at constant pressure, x the spatial 	coordinate and λ the thermal conductivity. Derive the stability condition for the time-explicit method, assuming uniform thermal conductivity and grid spacing, and a linearised source term S=SPTP+SC.  [17 marks]

Question 4

Explain the main differences between time-explicit and time-implicit discretisation schemes. [5 marks]