Question 1

(a) Let’s do the replacement S=-dpdx, where S is a constant source term. Integrating using the finite-volume method :

VddyμdudydV+VSdV=0,        [2 marks]

with dV=Ady , where A is the area which is a constant and cancels out:

snddyμdudydy+snSdy=0,

where s: south and n: north. In the first integral, dy can be eliminated and the integral of a differential is the function itself, whereas the integrand function in the second integral is a constant, such that :

μdudyn-μdudys+S∆y=0.       [3 marks]

Using a linear profile assumption and the fact that the viscosity is constant:

μdudyn= μuN-uPδyn,  μdudys= μuP-uSδys,           [3 marks]

Rearranging :

-μδysuS+μδys+μδynuP-μδynuN=S∆y.         [2 marks]

(b)

δy=∆y=Hn-1=0.005 m

Matrix A :                                                                           Matrix B :

Marking scheme : [1 mark] for correct dy, [1] for correct boundary conditions in A (1st and last row), [1] for correct procedure in calculating the 2nd, 3rd and 4th rows of A, [1] for correct boundary conditions in B (1st and last element), [1] for correct procedure in calculating the 2nd, 3rd and 4th elements of B. [1] if overall calculation is correct.

(c) Appropriate initial guess is ui(0)=0, where i=1,…,n. [1 mark]

First Gauss-Seidel loop [2 marks]:

CV1: u1=0

CV2: -0.2u1 +0.4u2 -0.2u30=5⟹u21=50.4=12.5

CV3: -0.2u21+0.4u3 -0.2u40=5⟹u31=5+0.2×12.50.4=18.75

CV4: -0.2u31+0.4u4 -0.2u50=5⟹u41=5+0.2×18.750.4=21.875

CV5: u5=0

Second Gauss-Seidel loop [1 mark]:

CV2: -0.2u1 +0.4u2 -0.2u31=5⟹u22=5+0.2×18.750.4=21.875

CV3: -0.2u22+0.4u3 -0.2u41=5⟹u32=5+0.2×21.875+0.2×21.8750.4=34.375

CV4: -0.2u32+0.4u4 -0.2u51=5⟹u42=5+0.2×34.3750.4=29.6875

Question 2

(a)

Let’s start by integrating the heat conduction equation for an internal control volume: 
VddxλdTdxdV=0,  [3]

with dV=Adx , where A is the cross-sectional area of the domain, which is a constant and cancels out:

weddxλdTdxdx=0,

where w: west and e: east. In the integral, dx can be eliminated and the integral of a differential is the function itself, such that :

λdTdxe-λdTdxw=0.    [2]

Using a linear profile assumption and the fact that the thermal conductivity and δx are constant:

λdTdxe= λTE-TPδx,  λdTdxw= λTP-TWδx.  [3]

Substituting in the equation above and rearranging :

λTE-TPδx-λTP-TWδx=0,         [2]

or:

-λδxTE+2λδxTP-λδxTW=0.

(b)

For the leftmost boundary control volume, centred at x=0, we have the dummy equation :

aBTB+aITI=b,  aB=1,aI=0, b=Ta.       [2]

For the rightmost boundary control volume, centred in x=L, we need to be careful to discretise the equation accounting for the knowledge of the heat flux. Considering a boundary control volume bounded by the internal face i at the left and the right face coinciding with the boundary node B:

iBddxλdTdxdx=0,    [2]

and:

λdTdxB-λdTdxi=0.

Now, based on the Fourier law:

λdTdx=-q,

but, in figure Q10, the heat flux is directed towards -x, and the exercise states that q>0, this means that:

λdTdxB=q.

Replacing in the equation above:

q-λdTdxi=0,

and :

q-λTB-TIδx=0.    [2]

Rearranging, we obtain the final equation:

-λδxTI+λδxTB=q.    [1]

(c)

δx=Ln-1=0.025 m.   [1]

The matrices look like [2]:

A=10000-λδx2λδx-λδx000-λδx2λδx-λδx000-λδx2λδx-λδx000-λδxλδx,   B=Ta000q

And, with λδx=250 W∙m-1∙K-1 0.025 m=10000 W∙m-2∙K-1, Ta=290 K, q=10000 W∙m-2:

A=10000-1000020000-10000000-1000020000-10000000-1000020000-10000000-1000010000,   B=29000010000

Question 3

Let’s do the replacement S=ρg, where S is a constant source term. Integrating using the finite-volume method:

VddxμdvdxdV+VSdV=0,        [2 marks]

with dV=Adx , where A is the area which is a constant and cancels out:

weddxμdvdxdx+weSdx=0,

where w: west and e: east. In the first integral, dx can be eliminated and the integral of a differential is the function itself, whereas the integrand function in the second integral is a constant, such that :

μdvdxe-μdvdxw+S∆x=0.       [3 marks]

Using a linear profile assumption and the fact that the viscosity is constant:

μdvdxe= μvE-vPδxe,  μdvdxw= μvP-vWδxw,           [3 marks]

Rearranging :

-μδxwvW+μδxw+μδxevP-μδxevE=S∆x.         [2 marks]

(b)

δx=∆x=Wn-1=0.0025 m

Matrix A :                                                                           Matrix B :

Marking scheme : [1 mark] for correct dx, [1] for correct boundary conditions at left boundary (1st row of A and B), [1] for correct boundary condition at right boundary (last row of A and B) [1] for correct procedure in calculating the 2nd, 3rd and 4th rows of A, [1] for correct procedure in calculating the 2nd, 3rd and 4th elements of B.

Question 4

To prove the convergence order of the discretisation scheme, we must evaluate the approximation of the derivative twice using different grid spacing, and compare this with the exact solution. The exact value of the first-order derivative in xe=1 is [1 mark]:

dTdxxe=3xe2+2xe2=5

Using δxe=0.2, we must set xP=0.9 and xE=1.1, so that δxe=xE-xP=0.2.

The values of temperature in those points are: TE=TxE=1.1=1.13+1.12+2=4.541 ;  TP=TxP=0.9=0.93+0.92+2=3.539 .

Therefore, the approximated value of the derivative is [2 marks]:

dTdxxe=TxE-TxPδxe=4.541-3.5390.2=5.01

Using δxe=0.4, we must set xP=0.8 and xE=1.2, so that δxe=xE-xP=0.4.

The values of temperature in those points are: TE=TxE=1.1=1.23+1.22+2=5.168 ;  TP=TxP=0.9=0.93+0.92+2=3.152 .

Therefore, the approximated value of the derivative is [1 mark]:

dTdxxe=TxE-TxPδxe=5.168-3.1520.4=5.04

Therefore, the error calculated as deviation between exact value of the derivative and its approximation is 0.01 when using δxe=0.2, and 0.04 when using δxe=0.4. The error becomes 4 times smaller when the grid spacing is halved, which proves that the discretisation scheme is 2nd order [1 mark].

Question 5

A penta-diagonal matrix is a matrix with 5 non-zero diagonals, while all the other elements are zero. The matrix of the coefficients is penta-diagonal in the case of a two-dimensional geometry [2 marks] discretised with a structured mesh [1 mark], because the temperature in a generic control volume within the domain will depend on the values at the 4 neighbours (north, east, south, west) [1 mark], thus giving a discretisation equation of the kind: aNTN+aWTW+aPTP+aETE+asTS=b [1 mark]

Question 6

A truncation error that is Oδx 2T ''' signifies that reducing the grid spacing by a factor  2, the error in the solution in principle reduces by a factor 4 [3 marks], and if the exact solution of the problem is a polynomial of order below 3, the discretisation scheme is exact [2 marks].

Question 7

To find the convergence order of the method, we must place the three points in a log-log plot dx vs E, and find the slope of the line that passes through these points. In a log-log plot, a line fitting our points has equation Y=m*X+q, with m being the slop and q the ordinate at which the line crosses the vertical axis. For us, Y is Y=log(E) where E is the error, and X=log(dx) where dx is the grid spacing, so that log(E)=m*log(dx)+q is the equation of the curve. We must find the slope m, we can do this by using 2 of the 3 points available, for example the first and third:

log(E1)=m*log(dx1)+q

log(E3)=m*log(dx3)+q

By subtracting the last equation from the first:

log(E1)- log(E3)=m*(log(dx1)- log(dx3))

And thus we can find m [3 marks]:

m=logE1E3logdx1dx3

Using the values given by the exercise, the equation above returns the value of m=3: this means that the numerical method is third-order accurate with respect to the grid spacing [2 marks]. Note that the result is independent of the choice of the logarithm base.